ESP32 Remote Sensor Project

# Software Setup Guide

Project: ESP32 + JSN-SR04T Ultrasonic Sensor + SSD1306 OLED Display

Document Type: Developer Handoff Reference

Prepared for: Matt

Date: July 13, 2026

Hardware Status: All wiring and connections assumed complete and verified

## Document Purpose

This guide covers the complete software configuration for the ESP32 + JSN-SR04T ultrasonic sensor + SSD1306 OLED display project. All hardware connections are assumed complete and working. This document is intended as a developer handoff reference covering IDE setup, library installation, firmware structure, sensor logic, display logic, threshold behaviour, and next development steps.

📄 Scope Note No hardware wiring or pinout details are included in this document. For wiring reference, consult the separate hardware diagram. This guide focuses exclusively on the software layer.

## 1. Required Tools

### 1.1 Arduino IDE

- Version: 2.x (recommended) or 1.8.x

- Download: arduino.cc/en/software

- Configure the IDE for ESP32 development before proceeding (see Section 1.2)

### 1.2 ESP32 Board Package (via Boards Manager)

Follow these steps to add ESP32 support to Arduino IDE:

- Open Arduino IDE → File → Preferences

- Add the following URL to the "Additional Board Manager URLs" field:

https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

- Go to Tools → Board → Boards Manager

- Search "esp32" → Install "esp32 by Espressif Systems"

- Recommended version: 2.x stable

- Board selection: Tools → Board → ESP32 Arduino → "ESP32 Dev Module" (or your specific board variant)

### 1.3 Upload Settings (Recommended Defaults)

| Setting | Value |
| --- | --- |
| Upload Speed | 921600 |
| CPU Frequency | 240 MHz |
| Flash Size | 4 MB |
| Partition Scheme | Default |
| Port (Windows) | COMx — select the port your ESP32 appears on |
| Port (macOS / Linux) | /dev/ttyUSBx or /dev/ttyACMx |

## 2. Required Libraries

Install all libraries via Arduino IDE → Sketch → Include Library → Manage Libraries.

### 2.1 Adafruit GFX Library

- Search: "Adafruit GFX"

- Author: Adafruit

- Purpose: Core graphics primitives (text, shapes, drawing) — used as a dependency by the SSD1306 library

- Install the latest stable version

### 2.2 Adafruit SSD1306

- Search: "Adafruit SSD1306"

- Author: Adafruit

- Purpose: Driver for the SSD1306-based OLED display over I2C

- Install the latest stable version

💡 Tip Installing Adafruit SSD1306 will prompt you to also install Adafruit GFX. Accept all dependency installs — both are required.

### 2.3 Wire (Built-in)

- Installation: Not required — included with the Arduino/ESP32 core

- Purpose: I2C communication between the ESP32 and the OLED display

📄 Note No additional libraries are required for the JSN-SR04T sensor. It uses standard GPIO digital I/O via the TRIG and ECHO pins — no special library needed.

## 3. Code Structure Breakdown

The project firmware is organized into three logical sections: Global Definitions, Setup, and Loop. Each section is documented below with annotated code and explanatory notes.

### 3.1 Global Definitions (top of file, before setup())

This section declares all pin assignments, constants, and library objects used throughout the sketch. All configurable values are defined here as macros for easy adjustment.

#include <Wire.h>
#include <Adafruit_GFX.h>
#include <Adafruit_SSD1306.h>
// --- OLED Display Config ---
#define SCREEN_WIDTH    128
#define SCREEN_HEIGHT   64
#define OLED_RESET      -1        // No reset pin used
#define SCREEN_ADDRESS  0x3C      // Common I2C address for SSD1306
Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, OLED_RESET);
// --- JSN-SR04T Sensor Pins ---
#define TRIG_PIN 5
#define ECHO_PIN 18
// --- Threshold Config ---
#define ALERT_THRESHOLD_CM 30    // Trigger alert if distance <= this value

⚠ Address Note SCREEN_ADDRESS may be 0x3C or 0x3D depending on your specific OLED module. If the display fails to initialize, change to 0x3D and retry. Use the I2C Scanner sketch in Section 7.3 to identify the correct address.

### 3.2 setup() Function

Runs once on boot. Responsible for initializing serial output, configuring sensor GPIO pins, and starting the OLED display. If the display fails to initialize, execution halts.

void setup() {
  Serial.begin(115200);
  // Sensor pin modes
  pinMode(TRIG_PIN, OUTPUT);
  pinMode(ECHO_PIN, INPUT);
  // Initialize OLED
  if (!display.begin(SSD1306_SWITCHCAPVCC, SCREEN_ADDRESS)) {
    Serial.println(F("SSD1306 allocation failed"));
    for (;;);  // Halt if display not found
  }
  display.clearDisplay();
  display.setTextSize(1);
  display.setTextColor(SSD1306_WHITE);
  display.setCursor(0, 0);
  display.println("System Ready");
  display.display();
  delay(1000);
}

💡 Tip The for(;;) halt on OLED failure is intentional — the display is essential to this project's output. Remove it only if you want the system to continue operating without a display (e.g., serial-only mode).

### 3.3 loop() Function

Runs continuously. Triggers the ultrasonic sensor, reads the echo duration, calculates distance, updates the OLED display, and outputs a debug line to Serial.

void loop() {
  long duration;
  float distanceCm;
  // --- Trigger Sensor ---
  digitalWrite(TRIG_PIN, LOW);
  delayMicroseconds(2);
  digitalWrite(TRIG_PIN, HIGH);
  delayMicroseconds(10);
  digitalWrite(TRIG_PIN, LOW);
  // --- Read Echo ---
  duration = pulseIn(ECHO_PIN, HIGH, 30000);  // 30ms timeout
  // --- Calculate Distance ---
  distanceCm = (duration * 0.034) / 2.0;
  // --- Update Display ---
  updateDisplay(distanceCm);
  // --- Serial Debug ---
  Serial.print("Distance: ");
  Serial.print(distanceCm);
  Serial.println(" cm");
  delay(500);  // Poll every 500ms
}

## 4. Sensor Logic — JSN-SR04T

The JSN-SR04T is a waterproof ultrasonic distance sensor that communicates via two digital GPIO pins: TRIG (trigger) and ECHO. No library is required — all interaction is direct GPIO.

### 4.1 How It Works

- Pull TRIG HIGH for 10 microseconds — this fires an ultrasonic pulse

- The sensor drives ECHO HIGH for a duration proportional to the pulse's round-trip travel time

- Measure the pulse width using pulseIn()

- Convert the duration to a distance using the speed of sound formula

### 4.2 Distance Formula

Distance (cm) = ( Echo Duration in µs × 0.034 ) / 2

- 0.034 cm/µs = speed of sound at room temperature (~343 m/s)

- Divide by 2 because the pulse travels to the target and back (round trip)

### 4.3 Reliable Operating Range

| Parameter | Value | Notes |
| --- | --- | --- |
| Minimum Range | ~20 cm | Objects closer than this may not return a clean echo |
| Maximum Range | ~450 cm | JSN-SR04T rated maximum |
| pulseIn() Timeout | 30,000 µs (30 ms) | Handles out-of-range without blocking indefinitely |

### 4.4 Edge Cases

- Duration returns 0 (timeout): Display "Out of Range" or "--" on the OLED — treat as no valid reading

- Erratic readings: Check for electrical interference or insufficient power supply decoupling near the sensor

- Very close objects (<20 cm): May read as 0 or produce unstable values — treat as invalid

## 5. Display Logic — SSD1306 OLED

The SSD1306 128×64 OLED communicates over I2C using the ESP32's SDA and SCL pins. The Adafruit SSD1306 library manages all low-level display communication through a frame buffer model — changes are staged in memory and pushed to hardware with a single display.display() call.

### 5.1 Display Update Function

void updateDisplay(float distance) {
  display.clearDisplay();
  display.setTextColor(SSD1306_WHITE);
  // --- Header ---
  display.setTextSize(1);
  display.setCursor(0, 0);
  display.println("Distance Sensor");
  display.drawLine(0, 10, 128, 10, SSD1306_WHITE);
  // --- Distance Readout ---
  display.setTextSize(2);
  display.setCursor(0, 18);
  if (distance <= 0 || distance > 450) {
    display.println("Out of Range");
  } else {
    display.print(distance, 1);  // 1 decimal place
    display.println(" cm");
  }
  // --- Alert Banner ---
  if (distance > 0 && distance <= ALERT_THRESHOLD_CM) {
    display.setTextSize(1);
    display.setCursor(0, 50);
    display.println("!! ALERT: TOO CLOSE !!");
  }
  display.display();  // Push buffer to screen
}

### 5.2 Key API Reference

| Function | Description |
| --- | --- |
| display.clearDisplay() | Clears the display buffer. Does not push to screen — must call display.display() to render. |
| display.display() | Pushes the current buffer to hardware. Required to show any changes. |
| display.setTextSize(1) | 6×8 px per character. setTextSize(2) = 12×16 px per character. |
| display.setCursor(x, y) | Sets text cursor position. x = pixels from left, y = pixels from top. |
| display.println() | Writes text to the buffer and advances the cursor to the next line. |
| display.print(val, 1) | Prints a float with 1 decimal place (second argument = decimal precision). |
| display.drawLine(x0, y0, x1, y1, color) | Draws a horizontal or diagonal line between two pixel coordinates. |

## 6. Threshold Logic

The alert threshold defines a proximity boundary. When a detected object is at or closer than the defined threshold distance, a visual alert banner is rendered on the OLED alongside the distance readout.

### 6.1 Configuration

#define ALERT_THRESHOLD_CM 30

Change this value to adjust the trigger distance. Default is 30 cm.

### 6.2 Logic Flow

| Condition | Display Output |
| --- | --- |
| duration == 0 OR distance > 450 cm | "Out of Range" — no alert shown |
| distance > 0 AND distance <= ALERT_THRESHOLD_CM | Distance value + "!! ALERT: TOO CLOSE !!" banner |
| distance > ALERT_THRESHOLD_CM AND distance <= 450 cm | Distance value only — normal operating state |

### 6.3 Extending: Multi-Zone Threshold

The threshold logic can be extended to support multiple proximity zones for finer-grained alerting:

// Example: multi-zone threshold
if (distance <= 15) {
  // Critical zone — immediate hazard
} else if (distance <= 30) {
  // Warning zone — approaching threshold
} else {
  // Clear zone — normal state
}

💡 Extension Idea Each zone could set a different display message, icon, or trigger an external output (buzzer, LED, WiFi notification). See Section 7.5 for suggested next steps.

## 7. Developer Handoff Notes

### 7.1 Project Summary

| Item | Detail |
| --- | --- |
| Microcontroller | ESP32 Dev Module |
| Sensor | JSN-SR04T Ultrasonic (waterproof) |
| Display | SSD1306 128×64 OLED (I2C) |
| IDE | Arduino IDE 2.x |
| Board Package | esp32 by Espressif Systems (v2.x stable) |
| Poll Rate | 500 ms — configurable via delay() in loop() |
| Alert Threshold | 30 cm — configurable via #define ALERT_THRESHOLD_CM |
| Serial Baud Rate | 115200 — must match Serial Monitor setting |

### 7.2 Key Customisation Points

- ALERT_THRESHOLD_CM — change the proximity alert trigger distance

- delay(500) in loop() — adjust the sensor polling frequency

- display.setTextSize() — adjust font size for different readout layouts

- pulseIn() timeout value — increase if environment requires longer detection range

- SCREEN_ADDRESS — change to 0x3D if OLED fails to initialize

### 7.3 Known Limitations

⚠ Known Limitations • JSN-SR04T minimum range is ~20 cm — objects closer than this may read as 0 or produce erratic values • OLED I2C address must match the physical hardware (0x3C or 0x3D) — verify with the I2C scanner below if needed • Serial.begin(115200) must match the Serial Monitor baud rate setting for readable debug output • No input debouncing or rolling average is implemented — single-sample readings may occasionally spike

### 7.4 I2C Scanner (Diagnostic Tool)

If the OLED fails to initialize, run this standalone sketch to identify the correct I2C address of your display module. Upload it separately, open the Serial Monitor at 115200 baud, and note the reported address.

#include <Wire.h>
void setup() {
  Wire.begin();
  Serial.begin(115200);
  Serial.println("I2C Scanner — scanning...");
  for (byte addr = 1; addr < 127; addr++) {
    Wire.beginTransmission(addr);
    if (Wire.endTransmission() == 0) {
      Serial.print("Found device at 0x");
      Serial.println(addr, HEX);
    }
  }
  Serial.println("Scan complete.");
}
void loop() {}

### 7.5 Suggested Next Development Steps

- Add WiFi or Bluetooth transmission of distance readings to a remote dashboard or mobile app

- Log readings to SD card or SPIFFS filesystem with timestamps for data analysis

- Add a buzzer alert on threshold breach for an audible warning output

- Implement a rolling average (e.g., last 5 readings) to smooth out noisy or erratic sensor values

- Add NTP time synchronisation and timestamp each reading for logging

- Implement multi-zone thresholds (critical / warning / clear) as outlined in Section 6.3

- Add deep sleep mode between readings to reduce power consumption for battery-powered deployments

Guide prepared for Matt's ESP32 Remote Sensor Project — Des Plaines, IL — July 13, 2026.
 Hardware setup complete. This document covers software configuration only.
 For hardware wiring and pin diagrams, refer to the companion hardware reference document.